Richmond American Framing Specs

- All main floor joists to be hangered from inside foundation using LB210AZ hangers

- Please figure 2x8 #2 Treated @ hangered walls, 2x6 elsewhere

- ¾ Edge Gold decking on all floors

- Walls to be figured as 16” oc, except at specified shearwalls as noted 12” oc

- If exterior walls are 18.5’ to 19’ figure 4x10 7/16 OSB lower level, 4x9 7/16 OSB upper.

- If exterior walls are 19.5’ to 20’ tall figure 4x10 7/16 OSB lower level and 4x10 7/16 OSB upper level.

- Figure exterior garage walls at 2x6 @ 16” oc, disregard 2x4 callout at garage walls

- 104-5/8 typical @ exterior garage walls with 4x10 7/16 OSB sheeting – Please figure garage studs separate from house studs.

- Figure interior shear wall panel sheeting as separate line.

- Figure stair stringers as 20’ if full run, figure as shorter lengths as necessary if there is a landing.

- Landing to be figured as ¾ Edge Gold U.N.O.

- Overhang Material LP Smart Side Panel (SS7648CP)

- Use 1” Shaftliner Densglass at inside of party walls U.N.O.

- Partywalls to be figured with C-channel at base of wall, hchannel between shaftliner vertically and horizontally. Acoustic Sealant, Fire Caulking, 1” Drive pins and ramset powder actuated shots. No c-channel; at top of wall.

- Material for the awnings should be figured as OU sku - #1 Doug Fir – S4S or RS depending on call out on plan – rather than Cedar as shown. Wall plate can be figured as 2x6 #1 DF.

- Please figure ½ RS SE T1-11 @ Awning ILO 1x6 T&G

- Over 20’ Tall Walls to be figured as 2x6 DF (OU sku)

- Please figure gable sheeting and roof sheeting as separate lines.

- Please figure 5/8” Densglass in its own section as called out for exterior fire details.

For questions please call Corey Boser 503-341-7420 or email corey.boser@bldr.com