Framing Specs:

- ¾ Edge Gold decking

- Figure walls 16” oc

- If exterior walls are 18.5’ to 19’ figure 4x10 7/16 OSB lower level, 4x9 7/16 OSB upper

- If exterior walls are 19.5’ to 20’ tall figure 4x10 7/16 OSB lower level and 4x10 7/16 OSB upper level

- Figure exterior garage walls at 2x6 @ 16” oc, disregard 2x4 callout at garage walls

- 116-5/8 typical @ exterior garage walls with 4x8 7/16 OSB sheeting

- No sheeting to be used @ interior garage walls except @ shear walls per plans

- All sill plate figured 2x6 including where joists hanger from plate

- Figure stair stringers as 20’ if full run, figure as shorter lengths as necessary if there is a landing

- Landing to be figured as ¾ Edge Gold U.N.O.

- Overhang Material LP Smart Side Panel (SS7648CP)

- Use Densglass only at inside of party walls U.N.O., 5/8” sheet rock to be figured by contractor @ conditioned space

Corey Boser 503-341-7420

I have been keeping my Microsoft Teams open on my computer so as long as I am in front of it I can be reached there.

Thank you!