Richmond American Siding Specs – T&I Siding

Please label housewrap pack material (housewrap, tape, window flashing tape, stinger staples, lath for window shims and window sealant) as HOUSEWRAP in comments.

Housewrap Pack:

One story house - 8 tubes of Dupont Sealant (DPWS) – 4 rolls House wrap Tape (TTAPE3-164)

Two story house - 14 tubes of Dupont Sealant (DPWS) – 6 rolls House wrap tape (TTAPE3-164)

One Pack of lath per house (RWLATH50)

Figure 475DFT for window sides and tops

Figure 975TFWNF for window sills

Stinger Staples (CH38SP for house wrap – 3 for 1 story home, 4 for 2 story home

Siding Pack

Figure 8” reveal siding callout as 8-1/4” Hardie (HZ10814CMSP)

Figure 5” reveal siding callout as 6-1/4” Hardie (HZ10614CMSP)

Figure Hardie Textured corners (OU) per plans

Figure Hardie Cedar Mill Panel at Soffits for front and back porches

Figure 12” Hardie Plank (HZ1012CMSP) for beam wrap anywhere not specified as  Hardie trim

Figure 2x2-20’ Primed White Wood at overhang for length of gables between outlookers

Figure 412JNTFLSHEA for butt joints – use (pieces of siding X .72) for quantity of joint flashing.

All trim called as 1x on plan on walls figure as 5/4x Hardie

All siding packs will need at least 1 piece of 1-1/2” High Back Z plus 1 piece for every 6 foundation vents (HU559408) – refer to Architectural Crawlspace Plan pages for foundation vent count

Front Doors and Windows see details per elevation – if detail calls for 2x over front windows, door and garage door figure 5/4x__ plus equal LF of 8-1/4 Hardie Plank and use 1-1/4” High Back White Z instead of 1” High Back Z

If details calls 1x3 over trim at front windows, door and garage door use 2-1/2” High Back White Z (321238HBFWHT)

For stone flashing please use 4”x2”x3/8” Brown Flashing (4238HMDB)

Use Tamlyn 5/16”x10’ Flashing for all Vertical Hardie Panel at meeting point (not necessary if Belly Band separates panels

Tail fascia not used -also please note that we will only be using 2820WWP, no 20820CYPP

Material for the awnings should be figured as OU sku - #1 Doug Fir – S4S or RS depending on call out on plan

Hardie Light Blocks – 2 at standard garage, 1 for garage service door, 1 if no covered patio @ back door, 2 additional for 3 car garage

2 Hardie Hose Bib Blocks per house

2 Hardie Receptacle Blocks per house

# 4 Plumbers Quickflash 1” per house (QFP100)

# Column Wraps per plan – most will be as follows but reference elevation specific detail:

1x6 Hardie for top collar

1x8 Hardie for sides and bottom collar

5/4x10 Hardie for sides

For 1’x1’ or 1’1” x 1’1” column details use 12” Hardie Plank with 1x4 Hardie Trim for corners.

SKU changes:

1275FF replaced with FWT1275

517317 (4X2-1/2X3/8 110D WHITE) replaced with 321238HBFWHT

TTAPE3-165 replaced with TTAPE3-164

PPG2212 is no longer available – please replace with 2220WWP – separate from Frieze Board material on list