Richmond American Siding Specs

Please label house wrap pack material (house wrap, tape, window flashing tape, stinger staples, lath for window shims and window sealant) as HOUSEWRAP in comments.

House wrap Pack:

- One story house - 8 tubes of Dupont Sealant (DPWS) – 4 rolls House wrap Tape (TTAPE3-164)

- Two story house - 14 tubes of Dupont Sealant (DPWS) – 6 rolls House wrap tape (TTAPE3-164)

- One Pack of lath per house (RWLATH50)

- Figure 475DFT for window sides and tops

- Figure 975TFWNF for window sills

- Stinger Staples (CH38SP for house wrap – 3 for 1 story home, 4 for 2 story home

- 3/8” Backer Rod for all windows and door openings

Siding Pack

- Figure 8” reveal siding callout as 8-1/4” Hardie (HZ10814CMSP) – we are not using 9-1/4”

- Figure 5” reveal siding callout as 6-1/4” Hardie (HZ10614CMSP)

- Figure Hardie Textured corners (OU) per plans

- Figure Hardie Cedar Mill Panel at Soffits for front and back porches

- Figure 12” Hardie Plank (HZ1012CMSP) for beam wrap anywhere not specified as Hardie trim

- Figure 2x2-20’ Primed White Wood at overhang for length of gables between outlookers (frieze board)

- Figure 2x2-20’ Primed White Wood for Inside Corner – please figure as separate line

- Figure 412JNTFLSHEA for butt joints – use (pieces of siding X .72) for quantity of joint flashing.

- All trim called as 1x on plan on walls figure as 5/4x Hardie with 1” High Back White Z Flashing

- All siding packs will need at least 1 piece of 1-1/2” High Back Z plus 1 piece for every 6 foundation vents (HU559408) – refer to Architectural Crawlspace Plan pages for foundation vent count.

- Front Doors and Windows see details per elevation – if detail calls for 2x over front windows, door and garage door figure 5/4x__ plus equal LF of 8-1/4 Hardie Plank and use 1-1/4” High Back White Z instead of 1” High Back Z

- If details calls 1x3 over trim at front windows, door and garage door use 2-1/2” High Back White Z (321238HBFWHT) and 1x3-12’ Hardie (HZU1312RB)

- For stone flashing please use 4”x2”x3/8” Brown Flashing (4238HMDB)

- Use 3/8” Z flashing Flashing for all Hardie Panel at meeting point (not necessary if Belly Band separates panels).

- Tail fascia not used -also please note that we will only be using 2820WWP, no 20820CYPP.

- Material for the awnings should be figured as OU sku - #1 Doug Fir – S4S or RS depending on call out on plan.

- Hardie Light Blocks – 2 at standard garage, 1 for garage service door, 1 if no covered patio @ back door, 2 additional for 3 car garage

- 2 Hardie Hose Bib Blocks per house

- 2 Hardie Receptacle Blocks per house

- 4 Plumbers Quickflash 1” per house (QFP100)

- Please figure corbel flashing for all corbels and overhang support details.

  - 3-1/2” sku - G383123812

  - 5-1/2” sku - G385123812

  - 7-1/4” sku - G387253812

  - Al others please figure as OU

# Column Wraps per plan – most will be as follows but reference elevation specific detail:

# Please notate items for post wraps as Post Wrap.

- 1x6 Hardie for top collar

- 1x8 Hardie for base collar

- 1x8 and 5/4x10 Hardie for sides

- For 1’x1’ or 1’1” x 1’1” column details use 12” Hardie Plank with 1x4 Hardie Trim for corners.

SKU changes:

- 1275FF replaced with FWT1275

- 517317 (4X2-1/2X3/8 110D WHITE) replaced with 321238HBFWHT

- TTAPE3-165 replaced with TTAPE3-164

- PPG2212 is no longer available – please replace with 2220WWP – separate from Frieze Board material on list

For questions please call Corey Boser 503-341-7420 or email corey.boser@bldr.com