Richmond American Siding Specs – T&I Siding

Housewrap Pack:

One story house - 8 tubes of Dupont Sealant – 4 rolls House wrap Tape (D13841455)

Two story house - 14 tubes of Dupont Sealant – 6 rolls House wrap tape (D13841455)

One Pack of lath per house

Figure 475DFT for window sides and tops

Figure 975TFWNF for window sills

Stinger Staples (CH38SP for house wrap – 3 for 1 story home, 4 for 2 story home

Siding Pack

Figure 8” reveal siding callout as 8-1/4” Hardie

Figure 5” reveal siding callout as 6-1/4” Hardie

Figure 412JNTFLSHEA for butt joints – use (pieces of siding X .72) for quantity of joint flashing.

All trim called as 1x on plan on walls figure as 5/4x

All siding packs will need at least 1 piece of 1-1/2” High Back Z plus 1 piece for every 6 foundation vents (HU559408)

Front Doors and Windows see details per elevation – if detail calls for 2x over front windows, door and garage door figure 5/4x__ plus equal LF of 8-1/4 Hardie Plank and use 1-1/4” High Back White Z instead of 1” High Back Z

If details calls 1x3 over trim at front windows, door and garage door use 2-1/2” High Back White Z (517317)

Light Blocks – 2 at standard garage, 1 for garage service door, 1 if no covered patio @ back door, 2 additional for 3 car garage

# 4 Plumbers Quickflash 1” per house (QFP100)

2 Hose Bib Blocks per house

2 Receptacle Blocks per house

Column Wraps per plan – most will be as follows but reference elevation specific detail:

1x6 Hardie for top collar

1x8 Hardie for sides and bottom collar

1x10 or 5/4x10 Hardie for sides

For 1’x1’ or 1’1” x 1’1” column details use 12” Hardie Plank with 1x4 Hardie Trim for corners.