We use 2x6 Treated plate at all hangers at foundation

Do not figure any eave trim on roof, only 1x3 and 2x8 Whitewood for Gables

All interior garage walls that are not shear walls do not require sheeting

All garage walls should be figure as 2x6

Exterior garage walls should be figured as 116-5/8” using 4x8 OSB for sheeting.

Elevation options: I have noticed that each estimator has their own way of doing elevation options so I would like to figure out a way to make all of them fairly uniform so I can have my ISR help me input these as needed. I would like to have all items that are the same on an elevation grouped as (ex.“Package Name” ABC) and all variables in each Elevation as its own group (ex. “Package Name” AB, “Package Name” C, “Package Name” AC, “Package Name BC, etc…)

Siding takeoffs: Please figure a housewrap pack (housewrap, tape, window flashing tape, stinger staples, lath for window shims and window sealant) as it’s own package and then figure individual siding packs for each elevation as most of them are different enough to warrant their own take off.

Corey Boser 503-341-7420

I have been keeping my Microsoft Teams open on my computer so as long as I am in front of it I can be reached there.

Thank you!