| k |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Builder Specifications |  |  |  |  |  |  |  |  |
| Builder Name: Richmond America |  |  |  |  |  |  |  |  |
| Salesperson: Corey Boser |  |  |  |  |  |  |  |  |
| Email: Corey.Boser@bldr.com |  |  |  |  |  |  |  |  |
| Sales Team: SalesTeamOne@buildersfirstsource.onmicrosoft.com |  |  |  |  |  |  |  |  |
| Confirm Plans with HCO |  |  |  |  |  |  |  |  |
| HCO Trumps Plan – Confirm with Corey if needed |  |  |  |  |  |  |  |  |
| Review all details on Plan |  |  |  |  |  |  |  |  |
| Stud Spacing |  |  |  |  |  |  |  |  |
|  | Standard: |  |  | 16” OC |  |  |
|  | Balloon Wall: |  |  | 16” OC |  |  |  |  |
|  | Detail - [W3-W6] |  |  | 12” OC |  |  |  |  |
| Junctions |  |  |  |  |  |  |  |  |
|  | Corners: |  |  | L-Type Junction |  |  |  |  |
|  | T Intersections: |  |  | L-Type Junctions |  |  |  |  |
| Typical Floor Heights |  |  |  |  |  |  |  |  |
|  | Main Floor Height: |  |  | See Elevation | 9-1-1/8 |  |  |  |
|  |  |  |  |  | 10-1-1/8 |  |  |  |
|  | Upper Floor Height: |  |  | See Elevation | 8-1-1/8 |  |  |  |
|  |  |  |  |  | 9-1-1/8 |  |  |  |
|  | Balloon wall height: |  |  | Depends of WH | 18-09-06 Typ. |  |  |  |
|  | Floor System: |  |  | 1-06-06 |  |  |  |  |
|  | Stairs wall height: |  |  | 13-09-14 Typ. |  |  |  |  |
| Floor System |  |  |  |  |  |  |  |  |
|  | Decking: |  |  | ¾” |  |  |  |  |
|  | Floor system: |  |  | 18” Confirm on plan |  |  |  |  |
| Opening Assembly Specs |  |  |  |  |  |  |  |  |
|  | Headers |  |  | Up |  |  |  |  |
|  | Cripples |  |  | Yes, with stud layout. |  |  |  |  |
|  | Flush cripples |  |  | Yes. |  |  |  |  |
|  | Trimmers |  |  | Trimmers all the same height |  |  |  |  |
| Opening Specs |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 6’ 8” |  | 8’ |  |
|  | Swing Door |  |  |  |  |  |  |  |
|  |  |  |  | Height | 6’ 11” |  | 8’ 3” |  |
|  |  |  |  | Width | +2” |  |  |  |
|  | Double Door |  |  |  |  |  |  |  |
|  |  |  |  | Height | 6’ 11” |  | 8’ 3” |  |
|  |  |  |  | Width | 2 X Door + 2” |  |  |  |
|  | Bypass Door |  |  |  |  |  |  |  |
|  |  |  |  | Height | 6’ 11” |  |  |  |
|  |  |  |  | Width | Exact |  |  |  |
|  | Byfold door |  |  |  |  |  |  |  |
|  |  |  |  | Height | 6’ 11” |  |  |  |
|  |  |  |  | Width | Exact |  |  |  |
|  | Header Height |  |  | Main | 8’ 0 1/8”~ |  |  |  |
|  |  |  |  | Upper | 7’ 0 1/8”~ |  |  |  |
| Header Heights can vary by an ¼” with varying header material. |  |  |  |  |  |  |  |  |
| Bathroom |  |  |  |  |  |  |  |  |
|  | RO for Insert |  |  |  | Depth |  | Width |  |
|  |  |  |  | Tub/Shower | 32” |  | 5’ | Per Plan |
|  |  |  |  | Shower | 36” |  | 5’ |  |
| Shear Callouts |  |  |  |  |  |  |  |  |
|  | 546 Detail – Extend header to end of Panel |  |  |  |  |  |  |  |
|  | 541 Detail – Sill Strap |  |  | (2) 2x windowsill, flat 2x6 blocking for 2 bays |  |  |  |  |
|  | Balloon blocking: |  |  | 8’ and 16’ |  |  |  |  |
|  | Adjoining Panels Edges |  |  | Add double studs every 4’ for 5 and 6 SW (confirm with SW Schedule) Make sure to include double cripple studs under windows at appropriate location. |  |  |  |  |
| Half Wall Heights |  |  |  |  |  |  |  |  |
|  | Standard |  |  | Height | 42” |  |  |  |
|  | Kitchen |  |  | Height | 2-10-04 |  |
|  | Under Stair |  |  | Height | 30” UNO |  | Look for cripple wall |  |
|  | VTP |  |  | None |  |  |  |  |
| Checklist |  |  |  |  |  |  |  |  |
|  | Garage Steps: 10” Drop on all Redwood Landing |  |  |  |  |  |  |  |
|  | When specified |  |  |  |  |  |  |  |

Richmond America – General

- Review HCO for doors.

- Radon Pipes leave a note if makes an interference

- Detail 541 Called out applies (2) 2x on sill and filler and increasing ¼” on window height. *** If calls out for a (2) CS strap the blocking will be double***

- Detail 541.0 only 1-ply sill and filler

- Remember to include all blocking according to detail called out on shower/tub.

- Make sure to check wall heights, when plans have a powder room in the front of the building.

Richmond America – Maple Grove

- Increase 1/8” on window heights

Richmond America - North Haven

- Do not include garage door.

- Make sure the master closet door is 4” aways from the master bedroom to fit electrical switches.

Richmond America – Feedback record

8-30-2024

- Full Length (back and front wall). - Balloon Stairs wall studs will go as “add from modeled only collection”. - Careful with balloon frame walls to start and finish between interior walls, compare with floor truss layout. - Look on HCO specification for open railing or half walls. - Kitchen island wall 2-10-04. - Fireplace opening cosmo or electric, Eli will create the opening. - Blockings smaller than the standard measures (1-02-08 or 1-10-08) will go as No.2. - Remember to look for cripple walls (under stairs) add a note on plan and elevation “Studs to be adjust on field”. - (2) studs for balloon frame walls, make sure to apply if requires. - Increase 1/8” on window heights only on Maple Grove subdivision. - Garage walls go through end to end.

11-22-2024

- Look for 906 detail where 4x6’s are called out.