Richmond American Siding Specs

Please label the house wrap pack material (house wrap, tape, window flashing tape, stinger staples, lath for window shims, and window sealant) as HOUSEWRAP in the comments.

House wrap Pack

- One-story house - 8 tubes of Dupont Sealant (DPWS) – 4 rolls of House wrap Tape (TTAPE3-164)

- Two-story house - 14 tubes of Dupont Sealant (DPWS) – 6 rolls of House wrap tape (TTAPE3-164)

- One Pack of lath per house (RWLATH50)

- Figure 475DFT for window sides and tops

- Figure 975TFWNF for window sills

- Stinger Staples (CH38SP for house wrap – 3 for one-story home, 4 for two-story home

- 3/8” Backer Rod for all windows and door openings

Siding Pack

Siding Material:

- Figure 8” reveal siding callout as 8-1/4” Hardie (HZ10814CMSP) – we are not using 9-1/4”

- Figure 5” reveal siding callout as 6-1/4” Hardie (HZ10614CMSP)

- Figure Hardie Textured corners (OU) per plan

- Figure Hardie Cedar Mill Panel at Soffits for front and back porches

- Figure 12” Hardie Plank (HZ1012CMSP) for beam wrap anywhere not specified as Hardie trim

- Figure 2x2-20’ Primed White Wood at overhang for length of gables between outlookers (frieze board)

- Figure 2x2-20’ Primed White Wood for Inside Corner – please figure as a separate line

Window Material:

- For the front doors and windows, see the details per elevation.

- 2x callout over front windows, door, and garage door: Figure 5/4x__ plus equal LF of 8-1/4 Hardie Plank and use 1-1/4” High Back White Z instead of 1” High Back Z

- 1 or 5/4x3 over trim at front windows, door, and garage door use 2-1/2” High Back White Z (321238HBFWHT) and 1x3-12’ Hardie (HZU1312RB)

Awnings & other structural siding materials:

- Tail fascia not used -also please note that we will only be using 2820WWP, no 20820CYPP.

- Material for the awnings should be figured as OU sku - #1 Doug Fir – S4S or RS depending on call out on plan.

Hardware:

- Figure 412JNTFLSHEA for butt joints – use (pieces of siding X .72) for quantity of joint flashing.

- All trim called as 1x on plan on walls figure as 5/4x Hardie with 1” High Back White Z Flashing

- All siding packs will need at least 1 piece of 1-1/2” High Back Z plus 1 piece for every 6 foundation vents (HU559408) – refer to Architectural Crawlspace Plan pages for foundation vent count.

- For stone flashing please use 4”x2”x3/8” Brown Flashing (4238HMDB)

- Use 3/8” Z flashing Flashing for all Hardie Panel at meeting point (not necessary if Belly Band separates panels).

- Hardie Light Blocks – 2 at standard garage, 1 for garage service door, 1 if no covered patio @ back door, 2 additional for 3 car garage

- 2 Hardie Hose Bib Blocks per house

- 2 Hardie Receptacle Blocks per house

- 4 Plumbers Quickflash 1” per house (QFP100)

- Please figure corbel flashing for all corbels and overhang support details.

  - 3-1/2” sku - G383123812

  - 5-1/2” sku - G385123812

  - 7-1/4” sku - G387253812

  - Al others please figure as OU

# Column Wraps per plan – most will be as follows but reference elevation specific detail:

# Please notate items for post wraps as Post Wrap.

- 1x6 Hardie for top collar

- 1x8 Hardie for base collar

- 1x8 and 5/4x10 Hardie for sides

- For 1’x1’ or 1’1” x 1’1” column details use 12” Hardie Plank with 1x4 Hardie Trim for corners.

For questions please call Corey Boser 503-341-7420 or email corey.boser@bldr.com