# 🧠 MH 1708 Estimating Workflow – AI Training Guide

This guide transforms your existing takeoff videos into structured AI-training lessons and standardized estimator workflows. Each module builds on the last to create a fully documented digital estimator process.

## 🎯 Overview

Purpose: Train AI systems and new estimators to understand, replicate, and optimize your full takeoff and pricing workflow using standardized logic, terminology, and structure.

Training Source Videos:

- Building the Second Floor – Framing & Design Insights

- Second Floor Joist System Overview and Planning

- Roof Takeoffs and Elevation Planning

- Inputting Pricing for the 1708 Exterior Unit

- MH 1708 Elevation A Siding Takeoff

Core Objectives:

- Standardize takeoff processes for repeatable accuracy.

- Train AI and staff to interpret patterns, materials, and plan logic.

- Reduce redundant manual entry and regenerate totals automatically.

- Establish a foundation for Power Automate and Excel/VBA integration.

## 🧩 Module 1 – Setup & Calibration

Focus: Preparing the workspace and plan environment before any measurement.

Goal: Ensure every takeoff starts from a calibrated, error-free foundation.

Tools: PlanSwift (Area & Linear Tools), Excel, Power Automate, SharePoint.

Steps:

- Confirm correct scale on import (⅛″ per ft default).

- Apply project template folder: 1708_Base_Template → Floors / Walls / Roof / Siding / Hardware.

- Verify naming conventions (Elevation A/B, Interior/Exterior Units).

- Remove unnecessary packages and export filter logic.

- Initialize ReadyFrame constant (0.45 default) in Excel for automated reference.

AI Tag Goals: scale_verification, naming_standardization, data_filtering, template_initialization.

Common Errors: Incorrect scale, duplicate package names, missing template linkage.

## 🏗️ Module 2 – Floor & Wall Framing

Focus: Constructing the second floor structure including joists, walls, and hardware.

Goal: Create accurate takeoffs for framing assemblies with automated quantity regeneration.

Key Elements:

- 2×12 floor joists @16” O.C.

- LSL rimboard integration.

- Blocking 48” O.C.

- MSTC40 straps, A35 connectors, LUS/HUS hangers.

Workflow Highlights:

- Start from architectural “S7” sheet – confirm joist type & hanger schedule.

- Use area tool for decking, linear for rimboard.

- Label blocking zones (2×12, 48” O.C.) and mark double joist conditions.

- Automate regeneration via macro or button-click event.

- Save framing logic as Component_Catalog entry (for future automation reuse).

AI Tag Goals: joist_logic, rimboard_validation, blocking_automation, hardware_insertion.

Common Errors:

- Rimboard mismatch (2×10 vs 2×12).

- Missing hangers in double-joist conditions.

- Manual recalculation of totals instead of automated regeneration.

## 🏠 Module 3 – Roof Framing & Sheathing

Focus: Calculating roof structure, fire-treated zones, and shear wall hardware.

Goal: Standardize roof takeoff logic, slope-based sheathing rules, and LS50 placement.

Core Data Points:

- Elevation A: 5:12 pitch (main) / 8:12 pitch (overbuild).

- Waste Factors: 15% standard / 20–25% for complex cuts.

- Fire-Treated Plywood: within 4 ft of property line.

- Gable sheathing: 4×8 / 4×9 panels @ 10 ft wall height.

Workflow Highlights:

- Separate Elevations A/B into unique roof packs.

- Apply correct pitch before measuring.

- Tag fire-treated areas; use color code for AI visibility.

- Insert LS50 straps (A=48” OC, B=30”, D=16”, F=8”).

- Confirm soffit type: vented vs solid (per plan proximity).

AI Tag Goals: roof_pitch_detection, fire_zone_mapping, soffit_type_classification, hardware_distribution.

Common Errors:

- Incorrect scale at roof plan.

- Unmarked fire-treated zones.

- Mixed soffit conditions in shared elevation packs.

## 🪚 Module 4 – Siding, Trim & Soffit

Focus: Measuring siding areas, trim runs, and soffit details for Elevation A.

Goal: Automate separation of material categories and apply standard waste logic.

Workflow Highlights:

- Verify scale and pitch alignment (8 ft wall, 10 ft gables).

- Area tool for SmartSide siding – apply 15% waste.

- Linear tool for trim (5/4×4, 5/4×6, fascia, corner boards).

- Separate vented vs solid soffits.

- Export totals into Excel categories:

  - Smart Lap Siding (SF)

  - Smart Trim (LF)

  - Smart Soffit (SF)

  - Accessories (EA)

AI Tag Goals: siding_area_measurement, trim_classification, soffit_segmentation, waste_factor_standardization.

Common Errors:

- Double-counted gables.

- Missed fire-rated soffit under 4 ft proximity rule.

- Trim overlap mismatch (e.g., 5/4×6 used instead of 5/4×4).

## 💲 Module 5 – Pricing Integration

Focus: Cleaning, consolidating, and pricing material exports.

Goal: Streamline manual entry and reduce redundancy across Exterior/Interior scopes.

Workflow Highlights:

- Export to Excel using “Use SKU to consolidate tallied items.”

- Filter by Exterior vs Interior scope.

- Apply formulas:

  - ReadyFrame = SquareFootage * 0.45

  - Ext Price = Qty * Unit Price

- Auto-check for missing unit prices (highlight blank cells red).

- Duplicate and adjust Interior Units sheet (apply 0.5x or 2x logic where needed).

AI Tag Goals: pricing_cleanup, scope_filtering, readyframe_calc, cost_validation.

Common Errors:

- Misapplied scope filters.

- Duplicate ReadyFrame constants.

- Missing hangers or hardware in export sheet.

## 🔍 Module 6 – QA & Continuous Improvement

Focus: Validation, optimization, and iterative AI training.

Goal: Establish automated QA loops to verify material accuracy and learn from corrections.

Verification Checklist:

- All elevations calibrated and named correctly.

- Hardware matches structural sheet (A35 → LS50 substitutions logged).

- Fire-treated sheathing only where required.

- Waste %s consistent across trades.

- No double-counted or orphan assemblies.

Automation Opportunities:

- Power Automate triggers on file export → auto QA summary.

- VBA macro: “Regenerate Totals + Color-Code Errors.”

- AI model retraining every 10 projects using logged corrections.

AI Tag Goals: error_detection, pattern_learning, process_refinement.

## 🧭 Next Steps

- Refine Inputs: Segment each video into 2–4 minute topic clips for micro-learning.

- Add Reference Tables: Include SKUs, hardware lists, and waste factors in a linked Excel knowledge base.

- Train the AI: Upload labeled datasets per module with voiceover transcription for contextual cues.

- Integrate Power Automate: Use Flow triggers to pull data from Excel exports for training reinforcement.

### ⚙️ Long-Term Vision

- Every takeoff video becomes an automated, teachable blueprint.

- The AI learns framing, siding, and pricing logic directly from your voice and cursor behavior.

- Estimators can interact with an AI assistant trained on your process, asking:

- “Show me the LS50 spacing logic for Elevation A roofs.” “What’s the ReadyFrame multiplier for the 1708 plan?”

Author: Corey Boser
Version: v1.0
Date: October 2025