# 🧠 MH 1708 Estimating Workflow – AI Training Guide (Trainer Notes Edition)

This enhanced version adds Trainer Notes for each module, aligning key timestamps, performance insights, and improvement prompts to support structured AI fine-tuning and estimator onboarding.

## 🎯 Overview

Purpose: Train AI systems and new estimators to understand, replicate, and optimize your full takeoff and pricing workflow using standardized logic, terminology, and structure.

Training Source Videos: 1. Building the Second Floor – Framing & Design Insights 2. Second Floor Joist System Overview and Planning 3. Roof Takeoffs and Elevation Planning 4. Inputting Pricing for the 1708 Exterior Unit 5. MH 1708 Elevation A Siding Takeoff

Trainer Focus: - Evaluate estimator speed, accuracy, and consistency across modules. - Highlight areas for automation or process improvement. - Annotate AI learning opportunities for visual, auditory, and behavioral cues.

## 🧩 Module 1 – Setup & Calibration

Video Reference: Second Floor Joist System Overview (0:00–1:45)

Trainer Notes: - Timestamp 00:10–00:30: Emphasize importance of setting correct scale early — include visual cue overlay for ⅛″ = 1′. - Timestamp 01:00–01:30: Reinforce naming convention steps (Elevation A/B) — recommend macro for auto-tagging imported plan sheets. - Improvement Cue: Use a quick template check before scaling to eliminate double handling. - AI Training Cue: Tag mouse movement near scale bar and verbal confirmation of scale for reinforcement.

Performance Metric: Setup time under 3 minutes, zero re-scaling corrections.

## 🏗️ Module 2 – Floor & Wall Framing

Video Reference: Building the Second Floor – Framing & Design Insights (00:00–08:00)

Trainer Notes: - Timestamp 00:45–01:30: Highlight efficient use of joist layout — consider pre-saved assemblies for 2×12 @16″ O.C. - Timestamp 02:00–03:00: Note rimboard validation — recommend VBA button for “check rimboard consistency.” - Timestamp 04:10–06:20: Reinforce blocking logic — show example macro for 48″ O.C. auto-fill. - Improvement Cue: Encourage batch labeling (joists, rimboard, blocking) before hardware placement. - AI Training Cue: Annotate cursor behavior and voice commands (“add blocking,” “verify hanger”).

Performance Metric: Maintain takeoff accuracy within ±1% for deck square footage.

## 🏠 Module 3 – Roof Framing & Sheathing

Video Reference: Roof Takeoffs and Elevation Planning (00:00–20:00)

Trainer Notes: - Timestamp 01:00–03:00: Reinforce dual roof elevation logic (A/B packs) — potential automation using roof-pack templates. - Timestamp 05:00–07:30: Tag scaling correction — trainer should pause video and emphasize why confirming scale first avoids rework. - Timestamp 08:00–12:00: Call out FRT plywood placement; introduce visual overlay for fire-treated zones. - Timestamp 16:00–19:00: LS50 spacing demonstration — add diagram in training overlay for A/B/D/F spacing differences. - Improvement Cue: Convert LS50 spacing logic into a formula-driven macro linked to roof type. - AI Training Cue: Pair voice and mouse input for pattern recognition of pitch, soffit type, and hardware placement.

Performance Metric: LS50 and A35 callouts match plan spec within 100% accuracy.

## 💲 Module 5 – Pricing Integration

Video Reference: Inputting Pricing for the 1708 Exterior Unit (00:00–48:00)

Trainer Notes: - Timestamp 00:00–02:00: Highlight export filter logic — show difference between interior vs exterior data cleanup. - Timestamp 03:30–08:00: Recommend pre-built Excel formula library for ReadyFrame, quantities, and extensions. - Timestamp 18:00–24:00: Note manual re-entry of repeat materials — suggest central “Material Map” reference workbook. - Timestamp 40:00–45:00: Verify consistency in unit pricing; trainer demonstrates Power Query merge of multiple takeoff sheets. - Improvement Cue: Introduce color-coded validation rule for missing pricing. - AI Training Cue: Label cursor hover events over pricing cells and verbal confirmations (“Exterior only,” “ReadyFrame at .45”).

Performance Metric: Manual entry time reduced by 30%; zero missing unit price cells.

## 🔍 Module 6 – QA & Continuous Improvement

Video Reference: Apply across all videos (post-export phase)

Trainer Notes: - Build a QA Summary Worksheet for every project: auto-log scale errors, naming mismatches, and hardware count variance. - Timestamp 05:00+ in each video: Capture AI learning moments where corrections occur (scale adjustment, hardware check). - Improvement Cue: Encourage on-screen error correction with commentary for reinforcement learning. - AI Training Cue: Flag corrections, highlight tool panels, and log before/after states for contextual model training.

Performance Metric: Zero missed hardware after final QA check; automated QA triggers generated via Power Automate.

## 🧭 Trainer Implementation Workflow

- Watch each recording once fully, then annotate timestamps with errors or inefficiencies.

- Capture 5–10 microlearning clips (2–4 minutes each) with specific improvement demonstrations.

- Link each clip to AI tag categories (joist_logic, roof_pitch_detection, pricing_cleanup, etc.).

- Export transcript snippets with visual annotation for dataset training.

- Review every 10 projects to retrain AI and update QA dashboards.

### ⚙️ Long-Term Vision

- Each training video doubles as an AI dataset and an onboarding module.

- Trainers refine efficiency per estimator by comparing timestamps and completion duration.

- The AI will eventually detect and correct errors during live takeoff sessions.

Author: Corey Boser
Version: v1.1 – Trainer Notes Update
Date: October 2025