Richmond American Siding Specs

Please label housewrap pack material (housewrap, tape, window flashing tape, stinger staples, lath for window shims and window sealant) as HOUSEWRAP in comments.

Housewrap Pack:

- One story house - 8 tubes of Dupont Sealant (DPWS) – 4 rolls House wrap Tape (TTAPE3-164)

- Two story house - 14 tubes of Dupont Sealant (DPWS) – 6 rolls House wrap tape (TTAPE3-164)

- One Pack of lath per house (RWLATH50)

- Figure 475DFT for window sides and tops

- Figure 975TFWNF for window sills

- Stinger Staples (CH38SP for house wrap – 3 for 1 story home, 4 for 2 story home

- 3/8” Backer Rod for all windows and door openings

Siding Pack

- Figure 8” reveal siding callout as 8-1/4” Hardie (HZ10814CMSP)

- Figure 5” reveal siding callout as 6-1/4” Hardie (HZ10614CMSP)

- Figure Hardie Textured corners (OU) per plans

- Figure Hardie Cedar Mill Panel at Soffits for front and back porches

- Figure 12” Hardie Plank (HZ1012CMSP) for beam wrap anywhere not specified as Hardie trim

- Figure 2x2-20’ Primed White Wood at overhang for length of gables between outlookers (frieze board)

- Figure 412JNTFLSHEA for butt joints – use (pieces of siding X .72) for quantity of joint flashing.

- All trim called as 1x on plan on walls figure as 5/4x Hardie with 1” High Back White Z Flashing

- All siding packs will need at least 1 piece of 1-1/2” High Back Z plus 1 piece for every 6 foundation vents (HU559408) – refer to Architectural Crawlspace Plan pages for foundation vent count

- Front Doors and Windows see details per elevation – if detail calls for 2x over front windows, door and garage door figure 5/4x__ plus equal LF of 8-1/4 Hardie Plank and use 1-1/4” High Back White Z instead of 1” High Back Z

- If details calls 1x3 over trim at front windows, door and garage door use 2-1/2” High Back White Z (321238HBFWHT)

- For stone flashing please use 4”x2”x3/8” Brown Flashing (4238HMDB)

- Use Tamlyn 5/16”x10’ Flashing for all Vertical Hardie Panel at meeting point (not necessary if Belly Band separates panels

- Tail fascia not used -also please note that we will only be using 2820WWP, no 20820CYPP

- Material for the awnings should be figured as OU sku - #1 Doug Fir – S4S or RS depending on call out on plan

- Hardie Light Blocks – 2 at standard garage, 1 for garage service door, 1 if no covered patio @ back door, 2 additional for 3 car garage

- 2 Hardie Hose Bib Blocks per house

- 2 Hardie Receptacle Blocks per house

- 4 Plumbers Quickflash 1” per house (QFP100)

# Column Wraps per plan – most will be as follows but reference elevation specific detail:

- 1x6 Hardie for top collar

- 1x8 Hardie for sides and bottom collar

- 5/4x10 Hardie for sides

- For 1’x1’ or 1’1” x 1’1” column details use 12” Hardie Plank with 1x4 Hardie Trim for corners.

SKU changes:

- 1275FF replaced with FWT1275

- 517317 (4X2-1/2X3/8 110D WHITE) replaced with 321238HBFWHT

- TTAPE3-165 replaced with TTAPE3-164

- PPG2212 is no longer available – please replace with 2220WWP – separate from Frieze Board material on list

For questions please call Corey Boser 503-341-7420 or email corey.boser@bldr.com