# STO Material Code System

Sales Team One - Descriptive Material Codes

Version 1.0 | December 2025 | Status: Active

# 1. Code Format Overview

The STO Material Code System uses human-readable codes that describe materials at a glance. Each code follows a consistent pattern that identifies the material category, dimensions, specifications, and any variants.

## 1.1 General Format

PREFIX-DIMENSION-SPEC[-VARIANT]

| Segment | Description |
| --- | --- |
| PREFIX | Material category identifier (2X4, OSB, HANGER, SID, etc.) |
| DIMENSION | Size, length, depth, or model number |
| SPEC | Species, grade, brand, or type specification |
| VARIANT | Optional treatment or variant suffix (-PT-GC, -TG, -FR) |

Example Codes:

| STO Code | Description |
| --- | --- |
| 2X4-8-SPF | 2x4 x 8' Spruce-Pine-Fir #2 |
| 2X6-92-5/8-DF | 2x6 x 92-5/8" Douglas Fir Stud Grade |
| 2X6-12-PT-GC | 2x6 x 12' Pressure Treated Ground Contact |
| HANGER-LUS28 | Simpson LUS28 Joist Hanger |
| OSB-7/16-4X8 | 7/16" OSB 4x8 Sheathing |
| GLB-5-1/2X18 | Glulam Beam 5-1/2" x 18" |

# 2. Dimensional Lumber

Dimensional lumber codes follow the format NXN-LENGTH-SPECIES where NXN is the nominal dimension.

## 2.1 Standard Lumber Prefixes

| Prefix | Nominal Size | Common Uses |
| --- | --- | --- |
| 2X4 | 2" x 4" | Studs, plates, blocking, light framing |
| 2X6 | 2" x 6" | Exterior walls, floor joists, rafters |
| 2X8 | 2" x 8" | Floor joists, headers, rafters |
| 2X10 | 2" x 10" | Floor joists, headers, beams |
| 2X12 | 2" x 12" | Floor joists, ridge boards, headers |
| 4X4 | 4" x 4" | Posts, columns, deck framing |
| 4X6 | 4" x 6" | Posts, beams, heavy framing |
| 6X6 | 6" x 6" | Posts, columns, structural support |

## 2.2 Length Codes

| Code | Length | Notes |
| --- | --- | --- |
| 8, 10, 12... | 8', 10', 12', etc. | Standard lengths in feet |
| 92-5/8 | 92-5/8" | Precut stud for 8' walls |
| 104-5/8 | 104-5/8" | Precut stud for 9' walls |
| RL | Random Length | Mixed lengths, priced per MBF |

## 2.3 Species Codes

| Code | Species | Characteristics |
| --- | --- | --- |
| DF | Douglas Fir | Strong, stiff, good for structural |
| SPF | Spruce-Pine-Fir | Economical, light, easy to work |
| HF | Hem-Fir | Good strength, moderate cost |
| SYP | Southern Yellow Pine | Dense, strong, takes treatment well |
| CED | Cedar | Naturally rot-resistant, exterior use |
| CYP | Cypress | Rot-resistant, primed exterior trim |
| RW | Redwood | Premium, naturally decay-resistant |

## 2.4 Complete Lumber Examples

| STO Code | Full Description |
| --- | --- |
| 2X4-8-SPF | 2x4 x 8' SPF #2 Standard |
| 2X4-92-5/8-DF | 2x4 x 92-5/8" Douglas Fir Stud Grade |
| 2X6-104-5/8-DF | 2x6 x 104-5/8" Douglas Fir Stud Grade (9' walls) |
| 2X6-16-SPF | 2x6 x 16' SPF #2 (Top Plates) |
| 2X10-20-DF | 2x10 x 20' Douglas Fir #2 (Headers/Joists) |
| 4X4-8-DF-PT-GC | 4x4 x 8' Douglas Fir Pressure Treated Ground Contact |
| 6X6-12-DF | 6x6 x 12' Douglas Fir #2 Post |

# 3. Engineered Lumber

Engineered lumber products use type-specific prefixes followed by dimensions and model information.

## 3.1 Engineered Lumber Prefixes

| Prefix | Product Type | Format | Example |
| --- | --- | --- | --- |
| IJT | I-Joist | IJT-DEPTH-MODEL | IJT-11-7/8-TJI110 |
| LVL | Laminated Veneer Lumber | LVL-THKxDEPTH | LVL-1-3/4X14 |
| LSL | Laminated Strand Lumber | LSL-THKxDEPTH | LSL-1-1/2X9-1/2 |
| GLB | Glulam Beam | GLB-WIDTHxDEPTH | GLB-5-1/2X18 |
| RIM | Rim Board | RIM-THKxDEPTH | RIM-1-1/8X9-1/2 |
| PSL | Parallel Strand Lumber | PSL-THKxDEPTH | PSL-3-1/2X11-7/8 |

## 3.2 I-Joist Models

| Model | Series | Typical Depths |
| --- | --- | --- |
| TJI110 | TJI 110 | 9-1/2", 11-7/8", 14", 16" |
| TJI210 | TJI 210 | 9-1/2", 11-7/8", 14", 16" |
| TJI230 | TJI 230 | 11-7/8", 14", 16" |
| TJI360 | TJI 360 | 11-7/8", 14", 16" |

# 4. Sheathing & Panels

## 4.1 OSB (Oriented Strand Board)

Format: OSB-THICKNESS-SIZE[-VARIANT]

| STO Code | Description |
| --- | --- |
| OSB-7/16-4X8 | 7/16" OSB 4x8 Wall Sheathing |
| OSB-15/32-4X8 | 15/32" OSB 4x8 Roof Sheathing |
| OSB-23/32-4X8-TG | 23/32" OSB 4x8 Tongue & Groove Floor |
| OSB-3/4-4X8-TG | 3/4" OSB 4x8 Tongue & Groove Subfloor |

## 4.2 Plywood

Format: PLY-THICKNESS-GRADE-SIZE[-VARIANT]

| STO Code | Description |
| --- | --- |
| PLY-1/2-CDX-4X8 | 1/2" CDX Plywood 4x8 |
| PLY-3/4-CDX-4X8 | 3/4" CDX Plywood 4x8 |
| PLY-3/8-CDX-4X8-PT | 3/8" CDX Plywood 4x8 Pressure Treated |

# 5. Hardware (Simpson Strong-Tie)

Hardware codes use category prefixes followed by the manufacturer's model number.

## 5.1 Hardware Prefixes

| Prefix | Category | Examples |
| --- | --- | --- |
| HANGER | Joist Hangers | HANGER-LUS28, HANGER-LUS210, HANGER-HUS210 |
| ANCHOR | Hold-Downs & Anchors | ANCHOR-HD5A, ANCHOR-HD2A, ANCHOR-ABA |
| BASE | Post Bases & Caps | BASE-ABU44Z, BASE-AC6Z, BASE-PBS44 |
| STRAP | Strap Ties | STRAP-LSTA21, STRAP-MST48, STRAP-ST6236 |
| CLIP | Clips & Angles | CLIP-A35, CLIP-H2.5A, CLIP-RTC42 |
| HDW | General Hardware | HDW-A34, HDW-CNW7 |

## 5.2 Common Joist Hanger Models

| STO Code | Model | Application |
| --- | --- | --- |
| HANGER-LUS26 | LUS26 | 2x6 Face Mount Hanger |
| HANGER-LUS28 | LUS28 | 2x8 Face Mount Hanger |
| HANGER-LUS210 | LUS210 | 2x10 Face Mount Hanger |
| HANGER-HUS210 | HUS210 | 2x10 Heavy Face Mount Hanger |
| HANGER-HGUS412 | HGUS412 | Girder Hanger 4x12 |

# 6. Siding & Exterior Products

## 6.1 Siding Prefixes

| Prefix | Product Type | Format |
| --- | --- | --- |
| SID-HZ-LAP | HardiePlank Lap | Horizontal lap siding |
| SID-HZ-PANEL | HardiePanel | Vertical panel siding |
| SID-HZ-SOFFIT | HardieSoffit | Vented/solid soffit panels |
| SID-LP | LP SmartSide | Engineered wood siding |

## 6.2 Exterior Trim (Hardie)

Format: TRIM-HZ-THICKNESSxWIDTH

| STO Code | Description |
| --- | --- |
| TRIM-HZ-5/4X4 | HardieTrim 5/4" x 4" |
| TRIM-HZ-5/4X6 | HardieTrim 5/4" x 6" |
| TRIM-HZ-5/4X8 | HardieTrim 5/4" x 8" |
| TRIM-HZ-5/4X12 | HardieTrim 5/4" x 12" (Fascia) |

# 7. Treatment & Variant Suffixes

Suffixes are appended to the base code to indicate special treatments or variants.

## 7.1 Treatment Suffixes

| Suffix | Meaning | Example |
| --- | --- | --- |
| -PT | Pressure Treated (general) | 2X6-12-DF-PT |
| -PT-GC | Pressure Treated Ground Contact (.40) | 4X4-8-DF-PT-GC |
| -PT-AG | Pressure Treated Above Ground (.15) | 2X4-8-SPF-PT-AG |

## 7.2 Panel/Sheathing Variants

| Suffix | Meaning | Example |
| --- | --- | --- |
| -TG | Tongue & Groove | OSB-23/32-4X8-TG |
| -RAD | Radiant Barrier | OSB-7/16-4X8-RAD |

## 7.3 Drywall Variants

| Suffix | Meaning | Example |
| --- | --- | --- |
| -FR | Fire Rated (Type X) | DRY-5/8-4X8-FR |
| -MR | Moisture Resistant (Green Board) | DRY-1/2-4X8-MR |

# 8. Specialty Items

## 8.1 Trusses

| STO Code | Description |
| --- | --- |
| TRUSS-FLR-1670 | Floor Truss Package for Plan 1670 |
| TRUSS-FLR-G914 | Floor Truss Package for Plan G914 |
| TRUSS-ROOF | Roof Truss (general) |

## 8.2 Vents & Accessories

| STO Code | Description |
| --- | --- |
| VENT-12X18 | 12" x 18" Gable Vent |
| VENT-18X24 | 18" x 24" Gable Vent |
| WRAP-TYVEK | Tyvek Housewrap |

# 9. Unified Code Cross-Reference

STO Material Codes integrate with the MindFlow Unified Coding System via the Item Type segment (IIII).

| Unified | Holt | Description | STO Prefixes |
| --- | --- | --- | --- |
| 1000 | 4085 | Framing/Lumber | 2X4, 4X4, GLB, IJT, OSB, PLY |
| 1200 | 4120 | Trusses | TRUSS |
| 2100 | 4155 | Siding/Exterior | SID, TRIM-HZ, WRAP |
| 2200 | 4156 | Roofing | ROOF, FLASH |
| 3000 | 4320 | Interior Trim | MILL, TRIM-INT |

# 10. Document Information

| Field | Value |
| --- | --- |
| Document Version | 1.0 |
| Last Updated | December 2025 |
| Status | Active - Production Reference |
| Total Materials | 516 unique SKUs with STO codes assigned |
| Related Documents | MindFlow Unified Coding System v3.0 |
| Owner | Corey - Project Lead |

--- End of STO Material Code System Reference ---